CEG 3136 Report

Lab 1

Submitted by Lab Group A01-12

Name

Joe Thozhuthumparambil

Ramzi Senouci

Faculty of Engineering

Experiment Date: Tuesday, September 17, 2024

Submission Date: Saturday, September 21, 2024

# Table of Contents

Table of Contents	2

Source code for gpio.c	3

Source Code for Alarm.c	4

Exploration Questions	8

Source code for gpio.c

/*

* gpio.c

*

*  Created on: Sep 22, 2025

*      Author: jthoz081

*/

// General-purpose input/output driver

#include "gpio.h"

// --------------------------------------------------------

// Initialization

// --------------------------------------------------------

// Enable the GPIO port peripheral clock for the specified pin

void GPIO_Enable(Pin_t pin) {

if (pin.port == GPIOA)

RCC->AHB2ENR |= RCC_AHB2ENR_GPIOAEN;

else if (pin.port == GPIOB)

RCC->AHB2ENR |= RCC_AHB2ENR_GPIOBEN;

else if (pin.port == GPIOC)

RCC->AHB2ENR |= RCC_AHB2ENR_GPIOCEN;

else if (pin.port == GPIOD)

RCC->AHB2ENR |= RCC_AHB2ENR_GPIODEN;

else if (pin.port == GPIOE)

RCC->AHB2ENR |= RCC_AHB2ENR_GPIOEEN;

}

// Set the operating mode of a GPIO pin:

// Input (IN), Output (OUT), Alternate Function (AF), or Analog (ANA)

void GPIO_Mode(Pin_t pin, PinMode_t mode) {

pin.port->MODER = ((pin.port->MODER) & ~(0b11 << (pin.bit * 2))) | (mode << (pin.bit * 2));

}

// --------------------------------------------------------

// Pin observation and control

// --------------------------------------------------------

// Observe the value of an input pin

PinState_t GPIO_Input(const Pin_t pin) {

return ((pin.port->IDR) >> pin.bit) & (0b1);

}

void GPIO_Output(Pin_t pin, const PinState_t state) {

if (state) {

pin.port->BSRR = (0b1 << pin.bit);

} else {

pin.port->BSRR = (0b1 << (pin.bit + 16));

}

}

void GPIO_Toggle(Pin_t pin) {

pin.port->ODR ^= (0b1 << pin.bit);

}

## Source Code for Alarm.c

/*

* alarm.c

*

*  Created on: Sep 28, 2025

*      Author: joe31

*/

#include <stdio.h>

#include "gpio.h"

#include "systick.h"

#include "alarm.h"

static AlarmState_t state = DISARMED;

static Time_t button_press_start = 0;

static Time_t toggle_led = 0;

static uint8_t button_pressed = 0;

static const uint16_t SHORT_PRESS_MS = 2000;  // <2s

static const uint16_t LONG_PRESS_MS = 3000;  // ≥3s

static const uint16_t TOGGLE_TIME_MS = 1000;  // 1 Hz

static const Pin_t BLUE_LED = { GPIOB, 7 };

static const Pin_t GREEN_LED = { GPIOC, 7 };

static const Pin_t RED_LED = { GPIOA, 9 };

static const Pin_t BUTTON = { GPIOC, 13 };

static const Pin_t MOTION = { GPIOB, 9 };

void Init_Alarm(void) {

// clocks

GPIO_Enable(BLUE_LED);

GPIO_Enable(GREEN_LED);

GPIO_Enable(RED_LED);

GPIO_Enable(BUTTON);

GPIO_Enable(MOTION);

// LED outputs

GPIO_Mode(BLUE_LED, OUTPUT);

GPIO_Mode(GREEN_LED, OUTPUT);

GPIO_Mode(RED_LED, OUTPUT);

// Input

GPIO_Mode(BUTTON, INPUT);

GPIO_Mode(MOTION, INPUT);

printf("%u ms : System initialized\n", TimeNow());

state = DISARMED;

GPIO_Output(BLUE_LED, 0);

GPIO_Output(GREEN_LED, 0);

GPIO_Output(RED_LED, 0);

printf("%u ms : System DISARMED\n", TimeNow());

GPIO_Callback(BUTTON, CallbackButtonPress, FALL);  // press = falling edge

GPIO_Callback(BUTTON, CallbackButtonRelease, RISE); // release = rising edge

GPIO_Callback(MOTION, CallbackMotionDetect, RISE); // motion = rising edge

}

void Task_Alarm(void) {

if (button_pressed) {

Time_t duration = TimePassed(button_press_start);

if (duration >= LONG_PRESS_MS) {

state = DISARMED;

GPIO_Output(BLUE_LED, 0);

GPIO_Output(GREEN_LED, 0);

GPIO_Output(RED_LED, 0);

printf("%u ms : System DISARMED\n", TimeNow());

}

}

if (state == ARMED) {

if (TimePassed(toggle_led) >= TOGGLE_TIME_MS) {

GPIO_Toggle(BLUE_LED);

GPIO_Toggle(GREEN_LED);

toggle_led = TimeNow();

}

}

}

void CallbackButtonPress(void) {

button_press_start = TimeNow();

button_pressed = 1;

}

void CallbackButtonRelease(void) {

if (!button_pressed)

return;

button_pressed = 0;

Time_t duration = TimePassed(button_press_start);

if (duration <= SHORT_PRESS_MS) {

state = ARMED;

GPIO_Output(BLUE_LED, 0);

GPIO_Output(GREEN_LED, 1);

GPIO_Output(RED_LED, 0);

printf("%u ms : System ARMED\n", TimeNow());

}

}

void CallbackMotionDetect(void) {

if (state == ARMED) {

state = TRIGGERED;

GPIO_Output(BLUE_LED, 0);

GPIO_Output(GREEN_LED, 0);

GPIO_Output(RED_LED, 1);

toggle_led = TimeNow();

printf("%u ms : System TRIGGERED\n", toggle_led);

}

}

## Exploration Questions

- It generates “Welcome to CEG3136!”; the place holders are formatted according to the arguments given.

- SysTick= (Clock Speed × Tick Period s)

- Question 3

  - The else handles rollover it can only count to (2^32) and the extra time passed is calculated in the else statement

  - The earliest time would be 2^32 = 12 days

  - To expose the problem, we can use a smaller sized datatype like uint8_t

- Question 4

  - The while loop takes more than 1 second to complete; 1 second from the delay and extra to modify registers.

  - The variable loopcount is incremented once per loop.

  - Using timepassed is ideal because it will free the cpu to do other things rather than blocking other processes to wait for certain time.

- We can use timepassed time stamps to calculate the time spend before toggling.

- Interrupt handlers are required to have short execution time because they interrupt the flow of the program to do higher priority tasks. USART messages are received abruptly; if we do not interrupt when a change is detected we might lose data.